# NU-Information Exchange System

# Computer Network – Lab 5D

# Sara Shakeel: 23F-0617

# Hajra Haseeb: 23F-0789

# Laiba Khalid: 23F-0784

# December 8, 2025

# Semester Project Self-Evaluation (Fall 2025)

| Name: Sara Shakeel | Name: Hajra Haseeb | Name: Laiba Khalid |
| --- | --- | --- |
| Reg. No.:23F-0617 | Reg. No.: 23F-0789 | Reg. No.: 23F-0784 |
| Contribution to the Project’s development: Developed the following modules: - Network topology - RIP routing | Contribution to the Project’s development: Developed the following modules: - Client code - Server code | Contribution to the Project’s development: Developed the following modules: - IP subnetting - Intercampus message routing - Report |

Project completed (in %): ____________________
Project Modules Successfully Implemented:

- TCP & UDP Socker Implementation

- Server Concurrency

- Inter-Campus Message routing

- Hybrid Protocol

Modules That Were Not Implemented:

- No Encryption

- No file transfer

Challenges faced & how they were overcome:

- CISCO Packet Tracer only allowed guest mode for three time. Solved by working offline

Learnings from the Project:

1. Real network stimulation

2. Understanding how Client server architecture works

3. TCP vs UDP application knowledge

Comments (if any):

The project stimulated our interest in the subject. It solidified our understanding of network protocols, routing concepts, and practical device configuration. In the next phase, we would explore security hardening, evaluate alternative routing protocols, and document performance benchmark

Contents

Part 1: Application Development	4

System Overview:	4

Architecture:	4

Protocol Strategy:	4

Multi-threading:	4

Key Features:	4

Custom protocol	4

TCP Protocol:	4

UDP Protocol:	4

Functional workflow	5

Connection Phase:	5

After Successful login:	5

Compilation and Run	5

Requirements:	5

Compilation:	5

Run:	5

Part 2: Network Architecture	6

Campus LAN Design	6

LAN Subnets table	6

WAN IP Addressing	6

RIP ROUTING	6

1.	RIP Routing at Islamabad Router (192.168.10.1)	6

2. RIP Routing at Lahore Router (192.168.20.1)	7

3. RIP Routing at Karachi Router (192.168.30.1)	7

4. RIP Routing at Peshawar Router (192.168.4.1)	7

Topology	8

# Part 1: Application Development

## System Overview:

The NU-Information Exchange System is a client-server application facilitating communication between FAST-NUCES campuses using a hybrid TCP/UDP protocol.

## Architecture:

- Central Server (Islamabad): Acts as the central hub managing all inter-campus communications

- Campus Clients: Lahore, Karachi, Peshawar, CFD, Multan - each operates as a client connecting to the central server

## Protocol Strategy:

- TCP (Port 8080): Reliable delivery for authentication and direct inter-campus messages.

- UDP (Port 8081): Efficient delivery for heartbeats (every 10s) and broadcast announcements

## Multi-threading:

- Server: Admin console (main), TCP accept thread, UDP listener, individual client handlers

- Client: User interface (main), heartbeat sender, TCP receiver, UDP receiver

## Key Features:

- Hard-coded credential authentication

- Intelligent message routing between campuses

- Real-time status monitoring via heartbeats

- System-wide broadcast announcements

- Department-level message addressing

## Custom protocol

### TCP Protocol:

Authentication:

Client => Server: Campus:<CampusName>,Pass:<Password>

Server => Client: AUTH:SUCCESS or AUTH:FAILED

Example: Campus:Lahore,Pass:NU-LHR-123

Direct Messaging:

Campus to campus messages

Client =>Server: TO:<TargetCampus>|DEPT:<Department>|MSG:<Content>

Server => Target:  FROM:<SourceCampus>|DEPT:<Department>|MSG:<Content>

Example: TO:Karachi|DEPT:Academics|MSG:Please send enrollment data

### UDP Protocol:

Heartbeat (Every 10 seconds):

Activity monitoring

Client => Server: HEARTBEAT:<CampusName>

Example: HEARTBEAT:Lahore

Broadcast:

System announcements

Server => All Clients: BROADCAST:<AnnouncementText>

Example: BROADCAST:System maintenance tonight at 2 AM

## Functional workflow

### Connection Phase:

- Client select campus and department

- Client opens TCP connection

- Immediately send authentication message

- Server validates credentials and responds with success/failure

### After Successful login:

- Client enable message controls

- Opens a UDP socket

- TCP listening thread starts

- UDP broadcast listening thread starts

- Heartbeat sending thread

- Server adds campus to active client list

## Compilation and Run

### Requirements:

- Qt Creator

- Qt widget module

- C++ 17 or later

- Server must run before clients connect

### Compilation:

- Create  two new Qt widget application

- Replace code for client and server in main.cpp files

- Configure the .pro file

### Run:

- Click build then run qmake

- Then build

- Then run server project and start server

- Indicator turns green, server listens on TCP 8080 and UDP 8081

- Run client project

- Input the number of client windows to open

- Click connect

# Part 2: Network Architecture

## Campus LAN Design

Each FAST NUCES campus has been designed as a separate Local Area Network (LAN).
Every campus LAN includes:

- 1 Router

- 1 Switch

- 2 PCs representing departments such as:

  - Admissions

  - Academics

  - IT

  - Sports

All PCs are assigned static IP addresses from their respective campus subnet, and the default gateway is the router’s LAN interface.

### LAN Subnets table

| Campus | Network ID | Router LAN IP | Admissions | Academics |
| --- | --- | --- | --- | --- |
| Islamabad | 192.168.1.0 | 192.168.10.1 | 192.168.10.2 | 192.168.10.3 |
| Lahore | 192.168.2.0 | 192.168.20.1 | 192.168.20.2 | 192.168.20.3 |
| Karachi | 192.168.3.0 | 192.168.30.1 | 192.168.30.2 | 192.168.30.3 |
| Peshawar | 192.168.4.0 | 192.168.40.1 | 192.168.40.2 | 192.168.40.3 |
| CFD | 192.168.5.0 | 192.168.50.1 | 192.168.50.2 | 192.168.50.3 |

## WAN IP Addressing

| Link | Router A | Router B |
| --- | --- | --- |
| Islamabad to Lahore | 10.0.0.1 | 10.0.0.2 |
| Islamabad to Karachi | 14.0.0.1 | 14.0.0.2 |
| Islamabad to Peshawar | 15.0.0.1 | 15.0.0.2 |
| Islamabad to CFD | 16.0.0.1 | 16.0.0.2 |
| Lahore to Karachi | 13.0.02 | 13.0.0.1 |
| Lahore to Peshawar | 12.0.0.2 | 12.0.0.1 |
| Lahore to CFD | 11.0.0.2 | 11.0.0.1 |
| Karachi to Peshawar | 18.0.0.1 | 18.0.0.2 |
| Karachi to CFD | 17.0.0.2 | 17.0.0.1 |
| Peshawar to CFD | 19.0.0.1 | 19.0.0.2 |

## RIP ROUTING

### RIP Routing at Islamabad Router (192.168.10.1)

### 2. RIP Routing at Lahore Router (192.168.20.1)

- To Islamabad (10.0.0.1)

- To Karachi (13.0.0.2)

- To CFD(11.0.0.2)

This allows Lahore to dynamically exchange routes with both Islamabad and Karachi.

### 3. RIP Routing at Karachi Router (192.168.30.1)

- 13.0.0.1 (to Lahore)

- 18.0.0.1 (to Peshawar)

- 17.0.0.2(to CFD)

RIP v2 allows Karachi to learn all remote subnets (Islamabad, Lahore, Peshawar,CFD).

### 4. RIP Routing at Peshawar Router (192.168.4.1)

Through RIP, Peshawar automatically receives routes to Islamabad, Lahore, and Karachi and CFD

## Topology